SHEDULE OF QUANTITIES FOR CIVIL AND SANITARY WORKS OF PROPOSED BUILDING OF HALL OF RESIDENCE - 04 NATIONAL INSTITUTE OF TECHNOLOGY AT-ROURKELA. ORISSA CONSULTANT :- M/S. DESIGN TECHNO KOUSTUV TOWER , 2ND FLOOR, BIKASH NAGAR, BADAMBADI, CUTTACK - 753012 ORRISA ABSTRACT COST ( FOR CIVIL & SANITARY WORKS) 1); CIVIL WORKS; Rs. 2); SANITARY WORKS; Rs.. TOTAL; Rs..

CIVIL WORK Sl. NO; DESCRIPTION; AMOUNT Rs.. 1; SECTION:I EARTHWORK IN EXCAVATION. 2; SECTION:II PLAIN/REINFORCED CEMENT CONCRETE WORK. 3; SECTION:III BRICK WORK. 4; SECTION:IV WOOD WORK. 5; SECTION-V STEEL WORK. 6; SECTION-VI FLOORING. 7; SECTION-VII PLASTERING. 8; SECTION-VIII ALUMINUM WORK. 9; SECTION-IX FINISHING WORK. 10; SECTION-X ROOFING. 11; SECTION-XI WATER PROOFING TREATMENT. 12; SECTION-XII MISCELLANEOUSITEMS. TOTAL. SECTION - I EARTH WORK EXCAVATION Sl .No; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Clearing site including removal of grass, bush and saplings of girth upto 3cm. measured at height of 1m. Above ground level and remove of rubbish up to distance of 50m. out side the area cleared; Sqm; 3800. 2; Earth work in excavation in foundation trenches or drains(not exceeding 10sqm.on plan) dressing of sides and ramming of bottoms, lift up to 1.5 m including getting out the excavated soil & disposal of surplus excavated soil as directed, within lead of 50m. i); All kind of soil.; cum; 800. ii); Hard Rock (without blasting)stack measurement will be followed; cum; 600. 3 i); Extra over excavating earth exceeding depth 1.50 m but hot exceeding 3.0 m.; Cum; 8. ii); Extra over excavating earth in hard rock exceeding 1.5m depth upto 3.0m depth.(Rate is over corresponding basic item for depth upto 1.5m); cum; 8. 4; Filling available excavated earth (excluding rock) in trenches, plinth, sides of foundations etc. in layers not exceeding 15 cm. in depth with watering, Lead upto 50 m and lift upto 1.5 m (Payment to be made on measurement of finished quantity).; cum; 922. 5; Filling in plinth by fine sand in layers not exceeding 150mm.and up to 750mm.as directed & consolidating same by thorough saturation with water ramming complete including the cost of supply of sand(Payment to be made on measurement of finished quantity; cum; 1266. Sl .No; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 6; Supplying chemical emulsion in sealed containers including delivering as diluting and injecting chemical emulsion for post constructional anti-termite treatment and creating a continuous chemical barrier at junction of wall and floor, along the external perimeter of building, expansion joints over the top surface of consolidated earth on which apron is to be laid, surroundings of pipes and conduits etc complete (plinth area of the building at ground floor(approx 521sqm) using Chlorpyriphos / Biflex emulsifiable concentrate 20% diluted to 1% concentration. The entire work is to be carried out as per specification laid down in relevant IS code of latest edition.; Sqm; 540. 7; Supplying chemical emulsion in sealed Containers including delivering diluting and injecting chemical emulsion for pre construction anti-termite treatment and creating a continuous chemical barrier and all round the column pits, wall trenches, under basement excavation, top surface of plinth filling, junction of wall & floor, along the external perimeter of building, expansion joints, over the top surface of consolidated earth on which apron is to be laid, surroundings of pipes and conduits etc. complete (plinth area of the building at ground floor (approx 2397 sqm) Chlorpyriphos / Biflex emulsifiable concentrate 20% diluted to 1% Concentration. The entire work is to be carried out as per specification laid down in relevant IS code of latest edition.; Sqm; 1850. Total carried to summary Rs. SECTION - II PLAIN /REINFORCED CEMENT CONCRETE WORK Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Providing and laying in position cement concrete of specified grade excluding the cost of centering and shuttering - All work upto Plinth level... a; 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 12mm nominal size); cum; 15. b; 1:4:8(1 cement:4coarse sand :8graded stone aggregate 40 mm nominal size); cum; 315. 2; Providing and laying damp proof course 40mm thick with cement concrete 1:2:4(1 cement : 2 coarse sand : 4 graded stone aggregate 12.5mm nominal size); Sqm; 400. 3; Extra for providing and mixing water proofing material in cement concrete work in the proportion recommended by the manufacturers.(Sika latex power.); Sqm; 400. 4; Providing and laying in position specified grade of reinforced cement concrete in foundation, footing, bases of columns, tie beam etc excluding the cost of centering, shuttering finishing and reinforcement - All work upto plinth level. a; 1:1.5:3(1 cement :1.5 coarse sand : 3 graded stone aggregate 20 mm nominal size); cum; 434. 5; Reinforced cement concrete work in beams suspended floors, roofs having slope upto 15 degree, landing ,balconies, shelves ,chajjas lintels, bands, plain window sills, staircases and spiral staircases upto floor five level excluding the cost of centering, shuttering, finishing and reinforcement with 1:1.5 :3 (1 cement1.5 coarse sand: 3 graded stone aggregate20 mm nominal size); cum; 1184. 6; Reinforced cement concrete work 1:1.5:3( 1 cement : 1.5 coarse sand : 3 graded stone aggregate 20 mm nominal size ) in walls (any thickness) including attached pilasters buttresses, plinth and string courses, fillets, columns pillars, piers, abutments, posts and struts, etc. upto floor five level excluding cost of centering, shuttering, finishing and reinforcement( Column ).; cum; 322. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 7; Precast reinforced cement concrete in shelves including setting in cement mortar 1:3(1cement : 3coarse sand) cost of required centering, shuttering & finishing with neat cement punning on exposed surfaces(excluding the cost of reinforcement) with 1:2:4(1cement : 2 coarse sand :4 graded stone aggregate 12.5mm nominal size).50mm thick before finishing; Sqm; 2580. 8; Providing & laying cement concrete 1:2:4 (1 cement : 2coarse sand : 4 graded stone chips 20mm nominal size) at all levels to window sills, couping etc. including cost of centering and shuttering etc complete; cum; 18. 9; Providing and fixing centering and shuttering including strutting, propping etc including providing and laying waterproof paper to make watertight shuttering & removal of form etc on completion of work in foundation, column shaft tie beam, etc. upto plinth level (only the area in contact with concrete to be measured) 9 to 12 mm. thick approved quality ply board shuttering in any concrete work; Sqm; 3648. 10; Providing & fixing centering & shuttering including strutting, propping etc, providing & laying water proof paper to make watertight shuttering & removal of form for concrete work at all levels above plinth. (only the area in contact with concrete to be measured) 9 to 12mm. thick approved quality ply board shuttering in any concrete work.; Sqm; 15834. 11; Extra for shuttering for additional height over4m upto 5.5mtr; Sqm; 35. 12; Reinforcement (grade Fe 415 / 500) for R.C.C work including straightening, cutting, bending position and binding all complete( Binding wires cost included ). i; T.M.T. bars ( Grade Fe 415 / 500 ); Mt; 208.00. ii; Mild steel (Grade Fe 415); Mt; 7.00. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 13; Providing and fixing M.S. Insert plate, angle etc. Of approved thickness and size, fixed to R.C.C. Slab, beam, column etc. including M.S. Lugs, necessary welding etc. as per drawing with two coats of synthetic enamel paint of approved shade and brand over a coat of zinc chromate primer to exposed surfaces all complete as per direction (weight to be calculated as per and standard tables no extra for weight of welding material and rolling margin shall be paid for.); Kg; 70.00. Total carried to summary Rs. SECTION – III BICK WORK Sl. No; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Brick work with bricks of class designation75 in foundation and plinth in cement mortar 1:6 (1 cement: 6 coarse sand).; Cum; 336. 2; Brick work (1:6) in superstructure above plinth level up to floor IV level; cum; 3154. 3; Half Brick masonry with class designation75 in superstructure above plinth level upto floor V level cement mortar 1:4 (1 cement:4 coarse sand), including supplying and laying two 6 mm ms steel bar in every three layers in all floors and heights as per drawing and specification complete.(Reinforcement cost to be included.); Sqm; 4094. 4; Providing & laying single brick flat soling with bricks with class designation50,including preparing the sub grade to proper level including ramming dressing, & filling joints with locally available sand complete in all respects; Sqm; 5. 5; Extra over item No. 1 above for forming circular hole in wall for fixing exhaust fans including cement plaster 1:4 to inner faces (approximately 300mm dia to 375mm. dia) at all level and heights; Each; 76. 6- a); Providing and laying 75mm thick brick work with brick of strength not less than 75 kg / Sqcm laid in cement mortar 1:3(1 cement: 3 coarse sand) in plain or curved wall etc complete in all respects including laying 1 no's 6 mm bar at every alternate layers in all floors and heights as per drawing and specification complete; Sqm; 150. b); Extra for 6 mm. steel.; Sqm; 150. Total carried to summary Rs SECTION - IV WOODWORK Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Providing & fixing pressed cold steel door frame made with 1.25mm thick CR Steel plate bend to shape as per drawing fixed to wall with M.S . Holdfast welded to frame, all complete including making good the wall & floor as per direction engineer in charge. Cost of hold fast& cost of concrete to be paid separately. Frame shall be coated with a coat of Coaltar paint & to be filled with cement concrete (1:2:4) mix.; RM; 1457. 2; Providing klin seasoned & chemically treated sal wood work in frames of doors and fixing in position including two coats of wood preservative applied to rear of frame in contact with masonry & concrete surface IS 12896 :1990); cum; 9. 3; Providing and fixing 35 mm thick flush door shutter with ISI marked non decorative type with solid block board construction with frame of first class hard wood and well matched. commercial veneering on both faces of shutter finished as per grains and cross bands on face veneers bonded with phenol formaldehyde synthetic resin on both faces of shutter finished as per specifications mentioned (Drawing) with teak wood lipping out of 6 mm thick. (Excluding the cost of fittings.); Sqm; 427. 4; Providing and fixing panelled or panelled and glazed shutters for doors, windows and clear storey windows including black enameled M.S. butt hinges with necessary screws including, paneling which will be paid for separately kiln seasoned and chemically treated second class hard( kasi ,tentra piasal ) wood 38 mm the shutter frame; sqm; 185. 5; Providing and fixing wooden paneling for doors windows and clerestory windows (Area of opening for panel inserts excluding portion inside grooves or rebates to be measured).Paneling for paneled shutters 25mm thick ,kiln seasoned chemically treated second class hard ( kasi tentra , piasal ) wood; Sqm; 5. 6; Providing and fixing wooden moulded beading to door and window frames with iron screws, plugs and priming coat on unexposed surface etc. complete.. a; second class teak wood 50x20 mm; Meter; 1790. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 7; Providing 40x3 mm flat iron hold fast 25 cm including fixing to frame with 10 mm diameter bolts, nuts and wooden plugs and embeddings in cement concrete block 30x10x15 cm 1:2: 4mix(1 cement: 2 coarse sand:4 graded stone aggregate 20mm nominal size); Each; 2150. 8; Providing and fixing bright finished MS butt hinges with necessary screws etc Complete 125 x 65 x 2.12 mm ( ordinary type) welded to door frame where required; Each; 1280. 9; Providing and fixing 50 mm bright finished stainless steel cup board or ward robe knob with necessary screws ( best make of approved quality); Each; 400. 10; Providing and fixing stainless steel handles 150 mm x10 mm conforming to IS: 4992 with necessary screws. Etc complete.; Each; 855. 11; Providing and fixing 75mm special quality bright finished stainless steel cupboard or wardrobe locks with four levers including necessary screws etc. complete (best make of approved quality.); Each; 500. 12; Providing and fixing bright finished stainless steel hanging type floor double door stopper with necessary screws etc. Complete; Each; 330. 13; Providing and fixing bright finished Brass hydraulic door closer (Best make of approved quality) with necessary accessories and screws etc .Complete; Each; 5. 14; Providing and fixing stainless steel tower bolt with necessary screws etc. complete. a; 150 x 10 mm; Each; 80. b; 200 x 10 mm; Each; 400. 15; Providing and fixing bright finished stainless steel 100mm Mortise latch and lock with six levers and a pair of anodized (anodic coating) not less than grade AC10 as per IS : 1868 aluminum lever handles with necessary screws etc complete ( Best make of approved quality); Each; 5. 16; 40mm dia. Rubber buffer fixed to wall with m.s. Screws; Each; 189. 17; Providing and fixing Godrej make door bolt having integrated locking arrangements( Kaadi Tala)with nuts and screws etc complete 250x 12mm; Each; 358. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 18; Providing and fixing m.s. Curtain hooks with coiled wire with end loops over wooden in walls to be fixed over window and cupboard; Each; 655. 19; Providing and fixing curtain rod 20 mm dia of 1.25 mm thick made of chromium plated stainless steel plate with wooden brackets fixed to the wall with stainless steel screw all complete; Meters; 26. 20; Providing & fixing 30mm moulded P.V.C. / fiber door with solid second class seasoned& chemically treated hard wood core having arrangement for fixing required fixtures Top moulding layer should be minimum 2mm. thick hot pressed over the core. Design, shade & quality(Dura sleek or equivalent make) should be approved Architect; Sqm; 150. 21; Providing & fixing 4.0mm clear glass in wooden Windows and Ventilators as per drawing and design complete to satisfaction of Engineer - in - Charge / Architects Rate include cost of glass, glass fixing putty , nails etc; Sqm; 15. Total carried to summary Rs. SECTION - V STEEL-WORK Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Providing and fixing ISI Marked steel glazed doors, Windows and ventilators of standard rolled steel section joints mitred and welded with 15x3mm lugs, 10 cm. Long, embedded in cement concrete blocks 15 x 10 x 10 cm Of 1:3:6 (1cement : 3 coarse sand : 6 graded aggregate20mm nominal size)or with wooden plugs and screws or rawl plug and screws or with fixing clips or with bolts and nuts as required, including providing and fixing of 4mm thick glass panes plain/pin headed with glazing clips & special metal sash putty of approved make complete , including two point nose handle of approved pattern with applying priming coat of approved steel primer, and also including oxidized m.s casement stays straight peg type) with necessary screws etc(300mm weighing not less than 200 gms.)all complete as per direction of engineer in charge Windows- side hung; Sqm; 823. 2; Providing and fixing M.S. grills of required pattern in frames of windows etc. with M.S flats, square or round bars etc. all complete.. a; Fixed to wall and sill.; kg; 67379. 3; Providing and fixing M.S. railing of required pattern with flats, square or round bars etc. along with floor shoe all complete.; kg; 431. 4; Providing and fixing M.S. hand rail made with 50 dia M.S. pipe fixed to M.S. balustrade by means necessary welding, including 100 mm length (exposed) post & base plate including two coats of synthetic enamel paint Luxol / Dulux of approved shade over a coat of zinc chromate primer to M.S .hand rail at all height & level all complete as in drawing and direction engineer-in-charge; Meter; 302. 5; Providing & fixing in position collapsible steel shutters with vertical channels 20x10x2mm and braced with flat iron diagonals 20 x 5 mm size with top and bottom rail of T-iron 40 x 40 x 6mm with 40mm dia. Steel pulleys complete with bolts, nuts locking arrangement, stoppers handles, including applying a priming coat of approved steel primer; Sqm; 5. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 6; Providing & fixing openable and foldable grill gates / shutter for entrances made of flats square or round with m.s. Handle, locking system (sliding bolt and floor bolt), necessary screws and hinges. Fixed to walls by welding with 15x3mm lugs, 10cms of 1:3:6(1 cement3 coarse sand : 6 graded stone aggregate20 mm nominal size), necessary nuts & m.s. parliament hinge or pivots as required complete including applying priming coat of approved steel primer; kg; 600. Total carried to summary Rs. SECTION - VI FLOORING Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; cement concrete flooring 1:2:4 (1 cement2 coarse sand : 4 graded stone aggregate ) finished with a floating coat of neat cement including , cement slurry, but excluding the cost of nosing of steps etc. complete 40mm thick with 20 mm nominal size stone aggregate.; Sqm; 2402. 2; Cement plaster skirting (up to 30 cm height) with cement mortar 1:3 (1 cement: 3 coarse sand ) finished with a floating coat of neat cement 18 mm thick; Sqm; 358. 3; Providing & laying ceramic tiles (Kajaria or equivalent make) conforming to IS : 13755 and of approved design approx 300x300x 8mm size to floor at all level over minimum 20mm thick cement mortar base1:4(1 cement : 4sand) laid with grey cement slurry & joints finished with approved grouting material( Sika/Laticrete); Sqm; 437. 4; Providing and fixing glazed ceramic tiles (kajaria or equivalent make) conforming to IS 13753 and of approved design /shade approx 200 mm x 300 mm x 5.5mm With75mm.x 200 mm. x 5.5 mm decorative border to skirting and dado with tiles fixing adhesive of approved make including cost thereof and providing and laying minimum 10 mm thick cement plaster 1:3 (1 cement : 3 sand ) as leveling course and jointed with white cement slurry including racking out joints in brick wall close, & deep chipping of concrete surfaces .; Sqm; 1430. 5 a); Kota stone slab flooring over 20 mm thick C.M. 1:4 ( 1 Cement : 4 Coarse sand ) (average) base and jointed with grey cement slurry mixed with pigment to match the including rubbing and polishing complete20 mm minimum thick. ( 550 mm x 550 mm); Sqm; 4370. b); Kotta stone in skirting/dado over 12mm C.M.(1:3); Sqm; 350. 6; Kota stone slabs 20mm thick in risers of steps , treads on 20mm (average) thick cement mortar 1: 31cement : 3coarse sand ) and jointed with grey cement slurry mixed with pigment to match the shade of the slabs, including rubbing & polishing complete(Single piece of size1800mm x 300mm and 1800 mm x150mm.; Sqm; 130. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 7; Providing and fixing 40 mm wide and 4 mm thick plain glass strips in joints of flooring with cement mortar 1:3 complete in all respects as per drawing; Mtr; 3000.00. 8; Providing & laying minim. 16 to 18mm thick plain white marble strip border of approved design & quality laid in conjunction with kota stone flooring over a bed of 20mm( minim. ) thick sand cement mortar 1:4 (1 cement: 4coarse sand) including pointing with white cement mortar 1:2(1cement : 2marble dust), rubbing & polishing complete as per pattern shown in drawings and direction all complete in all respects. .Approx 50mm to 100mm wide border. a); Rajnagar white plain marble; Sqm; 170. b); Udayapur Green marble; Sqm; 90. 9; Providing & fixing joint less vitrified porcelain tile 10mm thick Naveen / Kajaria or equivalent make of approved shade to floor with cement slurry over a bed of minimum 12mm thick1:4 ( 1cement : 4coarse sand)sand cement mortar including necessary 1:2:4(1cement 2coarse sand:4 graded stone aggregate12mm nominal size) cement concrete backing the match level with adjustment floor level, finishing joints with white cement mixed with pigment to match the shade of tiles all complete in all respect as per drawing and direction ( Mirror polish )size 600 x 600 mm; Sqm; 35. 10; Providing & fixing 10mm thick vitrified porcelain tile Naveen / kajaria or equivalent make of approved shade & brand to wall with cement slurry over a bed of minimum 12 mm thick 1:3 ( 1cement : 3 coarse sand ) sand cement mortar including finishing the joints with white cement mixed with pigment to match the shade of tiles all complete in all respect as per drawings and direction( Mirror Polish )size 600 x 600 mm( approx. ); Sqm; 5. Total carried to summary Rs. SECTION - VII PLASTERING Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Providing & laying cement plaster to interior surfaces of walls columns and beams along walls with sand cement mortar complete in all respects as required for completion of the work in all floors. cement plaster 1 : 6 with cement mortar ( 1 cement : 6 sand. a); 16 - mm thick plaster for out side; Sqm; 12520. b); 12 - mm thick plaster for inside; Sqm; 25724. 2; Providing minimum 6 mm thick cement plaster( 1:4) to ceiling beams, soffit of stair case, chajja projection and isolated column projected beams etc in all floors complete as directed but excluding the cost of forming drip course which will be paid under separate item; Sqm; 8500. 3; Providing and forming drip course of 30 mm wide x 12 mm. Projection in plaster to soffit of projected slab like chajja etc. in all floors at edges with cement mortar 1 : 3 complete as directed for completion of work .; Meter; 513. 4; Providing & mixing water proofing compound sika plastocrete or similar approved quality to the cement plaster work as per manufacturers Specification.; Ltr; 1305. 5; Providing and laying white cement based wall putty thickness over a plastered surface to prepare the surface even and smooth complete .(Birla,JK,Sika); Sqm; 10837. Total carried to summary Rs. SECTION - VIII ALUMINIUM WORK Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Providing and fixing anodized aluminum windows, ventilators of bronze or approved colour and 15 micron thickness ( mat satin finish ) and of extruded sections of 34 series conforming to IS : 733 and IS : 1285, anodised transparent or dyed to IS : 1868( Minimum anodic coating of grade AC15) fixed with rawl plugs and screws etc. at all levels including the cost of coupling members between windows and necessary fittings viz. Aluminum handles friction hinges etc .including providing and fixing5mm. thick clear /obscured float glass panes gaskets, glazing beads and sealing joints all-round on both faces between work / lintel and windows with silicon sealants of approved make provision for fixing glass panes from inside and EDPM / Neoprine weather seats etc all as per drawing at all levels and heights. i); Fixed (Overall portion treated as fixed.); Sqm; 10. ii); Extra over above for ( extra rate admissible for the openable portion only ) : side hung shutters; Sqm; 10. 2; Providing and fixing twin style door complete with fixed glazed portion manufactured from Bronze or any other approved colour anodised( 15 micron thickness ) extruded aluminum alloy section complete with 5 mm thick clear float glass for fixed portions, Neoprene dry set glazing gasket , snap in beveled metal glazing stops on both sides, top and bottom styles of extruded aluminum alloy sections of special design for twin style door, 12 mm thick plate glass for d doors, necessary hardware fixtures viz. double action floor spring , mortice lock flush bolt, chromium plated brass tubular handle as per drawing on both faces all complete at all levels and heights. i); Fixed portion; Sqm; 20. ii); Openable portion; Sqm; 20. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 3; Providing & fixing structural glazing with openings using aluminum section 75 mm x50 mm powder coated ( 15 micron thick ) of OEL or equivalent make fixed to walls / R.C.C. slab using 125 mm long HILTI / FISHER screws5 mm glazing glass ( Ocean blue colors) fixed to frame work with adhesive tape & sealed with silicon as per drawing & designing, friction stay of heavy quality ( Ebco or equivalent make ) to be used for openings . The entire work to be tested for water tightness as directed by Engineer-in-Charge / Architect; Sqm; 90. Total carried to summary Rs. SECTION - IX FINISHING Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; White washing with lime to give an even shade Old / New work ( two coats ) mixed with approved adhesive complete to the satisfaction of engineer-in-charge; Sqm; 53910. 2; Distempering with 1st quality acrylic washable distemper (ready made of approved manufacturer and of required shade and colour complete as per manufacturer's specification. Two coats on old / new work all complete up to the satisfaction of engineer-in-charge; Sqm; 6850. 3; Applying one coat of cement primer of approved and manufacture on wall surface; Sqm; 12150. 4; Finishing walls with water proofing cement paint of approved brand & manufacture and of required make to give an even shade new work (three coats) @ Rs.3.84 Kg /10 Sqm( snowcem plus or equivalent complete up to the satisfaction of Engineer-in-charge; Sqm; 524. 5; Applying priming coat with ready mixed zinc chromate yellow primer of proved brand and manufacture on steel galvanised on / steel works; Sqm; 1850. 6; with ready mixed pink or grey primer of approved and manufacture on wood work (hard and soft wood); Sqm; 1530. 7; Painting with ready mixed synthetic enamel paint of approved brand and manufacture in all shades to give an even shade. All complete up to the satisfaction of engineer in charge. a); steel work(two coats); Sqm; 1850. b); wood work(two coats); Sqm; 2312. 8; Providing and applying two coats of exterior acrylic. Emulsion paint 'weather shield' of ICI India Ltd. or equivalent of approved shade over a coat of primer of same material I.E 2:I (2 paint1 water) by volume at all levels including sand papering, cleaning and preparation of surface.; Sqm; 11053. 9; Painting(two coats) synthetic enamel paint of approved brand & of required shade on rain water, soil waste and pipes and fittings with a coat of zinc. Chromate yellow primer to new work as per direction and satisfaction of Engineer-in-charge.. i; 100mm dia.(external); Mtr; 275. ii; 150mm dia (external); Mtr; 20. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 10; Providing and applying Acrylic emulsion paint( two coats complete to satisfaction of engineer in-charge) of approved brand and manufacturer over a coat of primer as required to wall ceiling etc. all complete in all floors and heights complete; Sqm; 100. Total carried to summary Rs. SECTION - X ROOFING Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; supply fitting, fixing cast iron down pipes of approved quality in position with necessary clamps, nails including making hole in wall floor etc. and cutting trenches in any soil through masonry concrete structure etc. if necessary & mending good damages including jointing with jointing materials . complete100 mm dia.(external); Mtr.; 250. 2; Supplying fitting, fixing C.I. Specials of down pipes in position with clamps nails etc. with jointing materials including cutting holes in masonry or concrete, mending good damages complete.. a); C.I. Bend of down pipes100 mm dia (external); Each; 23. b); C.I. shoe of down pipes100 mm dia(external); Each; 23. Total carried to summary Rs. SECTION - XI WATER PROOFING Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Applying & grouting a slurry of cement using2.75 kg/Sqm of cement admixed with proprietary water proofing compound sika latex or equivalent confirming to relevant I.S. mixed as per manufacturers specification over the RCC slab including the cleaning the surface before treatments; Sqm; 515. 2; Applying and laying screed concrete on the top of the roof in 1:2:4 ( 1 cement : 2 sand: 4 stone chips12.5 mm. down) water proofing admixture SIKALATEX or equivalent confirming to relevant I.S. mixed as per manufacturers specification with slope of 1 in 100 and necessary haul ring as per direction of engineer-in-charge. Average thickness50 mm; Sqm; 570. 3; Filling the expansion joint with SIKAFLEX as one component PU sealant including a backup material like Thermo Cole inside the groove of expansion joints. The width to depth ratio of sealant should be 2:1. For 50mm. Gap; RM; 20. 4; Water proofing treatment to toilet slabs with elastic two pack acrylic polymer based water proofing system with glass fibre as intermediate member by using approved brand to existing surfaces etc. (Work should be carried out through specialised agency.); sqm; 224. 5; Water proofing treatment:- Providing & laying integral cement based water proofing treatment including preparation of surface as required for treatment of roofs, balconies terraces etc consisting the following operation. a); Applying & grouting a slurry of cement using2.75 kg/Sqm of cement admixed with proprietary water proofing compound sika latex or equivalent confirming to relevant I.S. mixed as per manufacturers specification over the RCC slab including the cleaning the. b); Laying cement concrete using broken bricks / brick bats 25 mm to 100 mm size with 50% cement motor 1:5 (1 cement, 5 course sand ) admixed with proprietary water proofing compo und conforming to IS : 2645 to required slope and treating similarly junction of walls up to 300mm height including rounding the junctions of walls and slabs. c); After two days of proper curing applying the second coat of cement slurry admixed with proprietary water proofing compound conforming to propriety I.S 2645. d); Finishing the surface with 20 mm thick joint less cement mortar of mix 1:4 water proofing compound confirming to I.S 2645 and finally finishing the surface with broken ceramic tiles close jointed in cement mortar (1:4) pointed with cement slurry and finished with proper slope etc; Sqm; 3200. e); The whole terrace so finished shall be flooded with water for an minimum period of two weeks for curing and for final test. All above operation to be donein order and as directed and specified by the engineer in charge. With avg. thickness of 120mm and minimum thickness at khurra as65mm(Work should be carried out through specialised agency.) water proof chemical to be used must be SIKA LATEX OR SIKA Rain tight. Total carried to summary Rs. SECTION - XII MISCELLANEOUS WORK Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Providing all material and labour for average125mm thick plinth protection made out fo with 25mm thick 1:2:4 cement concrete on top of 100 mm thick P.C.C. 1:4:8 laid in proper slope and compaction after dressing the surface & ramming the earth width around the building perimeter , complete as per direction of Engineer in Charge / Architect; Sqm; 521. 2; Lettering with Black Japan paint of approved brand and manufacture 2 cm letter; per letter per cm height; 1246. Total carried to summary Rs. SANITARY AND PLUMBING WORK Sl. NO; DESCRIPTION; AMOUNT Rs.. 1; SECTION-I SANITARY & PLUMBING WORK. 2; SECTION:II WATER SUPPLY. 3; SECTION:III DRAINAGE. TOTAL. SECTION - I SANITARY & PLUMBING WORKS Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; Providing and fixing water closet squatting pan(Indian type W.C. pan) with 100 mm sand cast iron P or S trap, 10 liter low level white P.V.C flushing cistern with manually controlled device(handle lever) conforming to IS : 7231 Parry ware / Hind ware /Seabird / Orient (Coral) with all fittings and fixtures complete including cutting and making good the walls and floors whenever required. i); White Vitreous china Long pan pattern W.C pan of size 580 mm...; Each.; 39. 2; Providing and fixing white vitreous china pedestal type water closet (European type W.C pan ) with seat and lid, 10 liter low level white P.V.C. flushing cistern with manually controlled device (handle lever), conforming to IS : 7231 Parry ware/Hind ware/Sea bird /Orient (Coral) with all fittings and fixtures complete including cutting and making good the walls and floors wherever required. i); W.C. Pan with ISI marked black solid plastic seat and lid; Each.; 6. 3; Providing and fixing white vitreous china wash basin of approved make with C.I. /M.S. brackets15 mm C.P. brass basin mixture. 32 mm C.P brass waste of standard pattern, including painting of fittings and brackets, cutting and making good the walls wherever required. i); Flat back wash basin size 550x400 mm with single 15 mm C.P. brass basin mixture.; Each.; 22. 4; Providing and fixing Salem stainless steel AN ISI304 (18/8) kitchen sink as per IS 13983 with C.I. brackets and stainless steel plug 40mm including painting of fitting and brackets cutting and making good the walls wherever required. i); Kitchen sinks without drain board. a) 610 x510 mm bowl depth 200 mm; Each.; 2. 5; Providing and fixing 600 x450 x 6 mm beveled edge mirror of superior glass (of approved quality) complete with 6 mm thick hard board ground fixed to wooden cleats with CAP. brass screws and washers complete; Each.; 62. 6; Providing and fixing 300 x 300 mm, 18mm thick polished marble shelf as per drawing with edges rounded off and fixed to the wall finishing all complete; Each.; 62. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 7; Providing and fixing C.P brass Robe hook (heavy type) with C.P. brass screws fixed to wooden door/brick wall all complete; Nos.; 400. 8; Providing and fixing C.P. towel rail complete with brackets fixed to wooden cleats with C.P brass screws. i); 600 x 20 mm size.; Each.; 45. ii); Towel ring 150 mm. dia; Each.; 85. 9; Providing and fixing toilet paper holder.. i); C.P. brass.; Each.; 2. 10; Providing and fixing soil, waste and vent pipes.. i); 110 mm. dia. Sand cast iron S&S pipe.; Meters.; 350. ii); 75 mm. dia. Sand cast iron S&S pipe.; Meters.; 80. 11; Providing and fixing bend of required degree with access door, insertion rubber washer 3mm thick, bolts and nuts complete. i); 110 mm. dia. Sand cast iron S&S; Each.; 6. ii); 75 mm. dia Sand cast iron S&S...; Each.; 2. 12; Providing and fixing plain bend of required degree.. i); 110 mm. dia. Sand cast iron S&S; Each.; 22. ii); 75 mm. dia. Sand cast iron S&S; Each.; 4. 13; Providing and fixing single equal plain junction of required degree with access door, insertion rubber washer 3mm thick, bolts and nuts complete. i); 110 x 110 x 110 mm. dia. Sand cast iron S&S; Each.; 85. ii); 75 x 75 x 75 mm. dia sand cast iron S&S.; Each.; 5. 14; Providing and fixing terminal guard. ( Cowl ). i); 110 mm. dia. Sand cast iron S&S; Each.; 16. ii); 75 mm. dia. Sand cast iron S&S .; Each.; 3. 15; Providing lead caulked joints to sand cast iron / centrifugally cast (spun) iron pipes and fittings of diameter. i); 110 mm. dia.; Each.; 450. ii); 75 mm. dia.; Each.; 75. 16; Providing and fixing sand cast iron trap of self cleansing design with sand cast iron screwed down or hinged grating with or without vent arm complete, including cost of cutting and making good the walls and floors. i); 100 mm. inlet and 100 mm. outlet.; Each.; 105. ii); 75 mm. inlet and 75 mm. outlet.; Each.; 22. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 17; Cutting chases in brick masonry walls for following diameter sand cast iron pipes and making good the same with cement concrete1:3:6(1 cement : 3 coarse sand : 6 graded stone aggregate 12.5 mm nominal size) including necessary plaster and pointing in cement mortar1:4 (1cement : 4 coarse sand).. i); 110 mm. dia.; Each.. 18; Painting sand cast iron/soil, waste vent pipes and fitting with two coats of synthetic enamel paint of any colour such as chocolate grey or buff etc. over a coat of primer(of approved quality) for new work. i); 110 mm. dia.; Meters.. ii); 75 mm. dia.; Meters.. 19; Providing and fixing stainless steel jail (grating) of approved quality with or without hole for waste pipe for floor.. i); 125 mm dia. C.P. grating (heavy duty); Each.. ii); 150 mm dia. C.P. grating (heavy duty); Each.. iii); 100 mm dia. C.P. grating (heavy duty); Each.. 20; Providing & fixing sop disc of best available local quality of approved make and manufacture as per Engineer in charge or Architect; Each. Total carried to summary Rs.653,234.80 SECTION - II WATERSUPPLY Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 1; PIPES (TATA MEDIUM) AND FITTINGS :-. i); Providing and fixing G.I. Pipes (TATA medium or equivalent) complete with G.I. Fittings in pumps, including cutting and making good the walls etc. (internal work).Exposed on wall.. a); 80 mm. dia.; Meters.; 9. b); 25 mm. dia. Nominal bore.; Meters.; 50. c); 32 mm. dia. Nominal bore.; Meters.; 55. d); 40 mm. dia. Nominal bore.; Meters.; 130. e); 50 mm. dia. Nominal bore.; Meters.; 90. f); 75 mm. dia. Nominal bore.; Meters.; 75. ii); Concealed pipe including painting with anticorrosive bit mastic paint, cutting chases and making good the wall. a); 15 mm. dia. Nominal bore.; Meters.; 280. b); 20 mm. dia. Nominal bore.; Meters.; 65. c); 25 mm. dia. Nominal bore.; Meters.; 115. 2; BRASS FITTING :. Providing and fixing gun metal gate valve (with ISI mark) with C.I. Wheel of approved quality (screwed end).. i); 25 mm. dia. Nominal bore.; Each.; 2. ii); 32 mm. dia. Nominal bore.; Each.; 3. iii); 40 mm. dia. Nominal bore.; Each.; 32. iv); 50 mm. dia. Nominal bore.; Each.; 10. v); 63 mm. dia. Nominal bore.; Each.; 2. vi); 75 mm. dia. Nominal bore.; Each.; 4. 3; Providing and fixing C.I. Sluice valves (with cap) complete with bolts, nuts, rubber insertions etc (the tail pieces if required will be paid separately). i); 80 mm. diameter.. a); Class I .; Nos.; 1. 4; Painting G.I. pipes (TATA medium or equivalent) and fittings with white paint over a ready mixed priming coat, both of approved quality for new work. i); 15 mm. diameter pipe.; Meters.; 280. ii); 20 mm. diameter pipe.; Meters.; 65. iii); 25 mm. diameter pipe.; Meters.; 165. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. iv); 32 mm. diameter pipe.; Meters.; 55. v); 40 mm. diameter pipe.; Meters.; 130. vi); 50 mm. diameter pipe.; Meters.; 90. vii); 65 mm. diameter pipe.; Meters.; 75. viii); 80 mm. diameter pipe.; Meters.; 9. 5; Providing and fixing C.P. brass bib cock of approved quality conforming to IS:8931. i); 15 mm. dia. Nominal bore.; Each.; 45. 6; Providing and fixing C.P. brass angel valve for cistern, basin mixer and geyser points of approved quality conforming to IS:8931. i); 15 mm. dia. Nominal bore.; Each.; 190. 7; Providing and fixing C.P. connector of approved make; Each.; 190. 8; Providing and fixing shower set comprising of 1 no. C.P. shower rose, 15mm dia. C.P. Inclined shower arm and 15mm dia.C.P.wall mixer etc complete.; Each.; 45. Total carried to summary Rs. SECTION - III DRAINAGE: Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. STONE WARE PIPES AND FITTINGS. 1; Providing laying and jointing glazed stoneware pipes grade 'A' with stiff mixture of cement mortar in the proportion of 1:1 (1 cement:1fine sand) including testing of joints etc. complete.. i); 150 mm. dia.; Meters.; 500. ii); 100 mm. dia.; Meters.; 100. 2; Providing and laying cement concrete 1:5:10 (1 cement : 5 coarse sand : 10 graded stone aggregate 40mm nominal size) upto haunches of S.W. / P.V.C. pipes including bed concrete as per standard design. i); 150 mm. dia.; Meters.; 500. ii); 100 mm. dia.; Meters.; 100. 3; Providing and fixing square-mouth S.W. gully trap grade "A" complete with C.I. grating brick masonry chamber with bricks of class designation 75 and water tight C.I. cover with frame of 300x300mm size (inside) the weight of cover to be not less than 4.50kg and frame to be not less than 2.70 kg as per standard design.. i); 180 x 150 mm size P type.. a; With class100. Brick.; Each.; 20. 4; Constructing brick masonry manhole with 75 class designation brick in cement mortar 1:4 (1 cement : 4 coarse sand) R.C.C Top slab with 1:2:4mix (1cement : 2 coarse sand : 4graded stone aggregate 20mm nominal size),foundation concrete1:4:8: mix (1cement :4 coarse sand :8 graded stone aggregate 40mm nominal size) inside plastering 12mm thick with cement mortar1:3 (1 cement : 3 coarse sand) finished with floating coat of neat cement and making channels in cement concrete 1:2:4 (1 cement :2 coarse sand : 4 graded stone aggregate 20mm nominal size) finished with a floating coat o neat cement complete as per standard design. i); Inside size 90 x 80cm and 100cm deep including C.I. cover with frame(light duty) 455 x 610 mm internal dimensions total weight of cover and frame to be not less than 38 Kg (weight of cover23kg & weight of frame15 kg).. a); With class100 bricks.; Each; 2. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. ii); Inside size 90 x 80 cm and 100 cm deep including C.I. cover with frame (light duty) 455 x 610 mm internal dimensions t otal weght of cover and frame to be not less than 38 kg(weight of cover 23 kg and weight of frame 15 kg).. a); With brick with class designation 100.; Each.; 20. iii); Inside size 120 x 90 cm and 100 cm deep including C.I. Cover with frame (medium duty) 500 mm internal diameter, total weight of cover and frame to be not less than 116 kg (weight of cover 58 kg and weight of frame 58 kg; Each.; 8. 5; Extra for depth for manholes.. i); With F.P.S. brick class designation 100.; Meters.; 1. 6; Providing and laying non-pressure NP2 class(light duty) R.C.C. pipes with collars jointed with stiff mixture of cement mortar in the proportion of 1 : 2 (1 cement : 2 fine sand) including testing of joints etc. complete. i; 25mm dia R.C.C pipe; Meters; 250. 7; Providing and fixing in position precast R.C.C. manhole cover and frame of required shape and approved quality. i); Circular shape 560 mm internal diameters.; Each.; 30. 8; Construction of 200 users septic tank with chlorinating chamber complete as per drawing including earthwork, brickwork, concrete work, R.C.C. slab, plaster with neat finish,150 mm dia stone ware Tee etc all complete; Each.; 3. 9; Making soak pit 2.5 m diameter 3.0 meter deep with 45 x 45 cm dry brick honey comb shaft with brick of class designation 75 and S.W. drain pipe 100 mm. diameter, 1.8 m long complete as per standard design. I); With F.P.S. Bricks.; Each.; 3. 10; Constructing brick masonry open surface drain with brick of class designation 75 in cement mortar 1:4 (1cement : 4 f fine sand) including earth excavation, 10 cm thick bed concrete 1 :5 to (1 cement : 5 fine sand : 10 graded stone aggregate 40mm nominal size) and 25mm thick cement concrete 1:2:4(1 cement :2coarse sand : 4 graded stone aggregate 12.5mm nominal size) for filling haunches including 12mm cement plaster 1:4. (1cement :4coarse sand) with a floating coat neat cement inside the drain, its top and exposed. Sl.NO; DESCRIPTION; UNIT; QUANTITY; RATE Rs.; AMOUNT Rs.. 10; side including disposal of surplus earth complete as per standard design15 cm drain 20 cm average depth with. i); F.P.S. brick. 25 cm drain 30 cm average depth with; Meters.; 150. ii); F.P.S. brick. Providing and fixing P.V.C. waste pipe for sink; Meters.; 420. 11; or wash basin including P.V.C. waste fittings complete. Semi rigid pipe. i); 32 mm. dia.; Each.; 75. Total carried to summary Rs. Approved List of Manufacturers/ Brand for Civil Works Approved List to be used in works shall conform to Indian Standards Specification as published from time to time (and in the absence thereof as approved by Engineer-in –charge) in general the mode of measurement of the Civil engineering works shall be guided by I.S.I code No-1964(Revised) for Indian standard Method of measurement of building work. Sl. No; DESCRIPTION; I.S CODE; APPROVED MAKE. 1.; Cement : Ordinary port land cement 43 grade; 269:1967; OCL. 2.; Steel: a) mild steel:; 432(Part1)1982; TATA/SAIL. b) High strength deformed steel:; 1786:1985; TATA/SAIL. 3.; Sand:; 383:1963or 515 (Revised). 4.; Course Aggregate; 383. 5.; Lime:; 792:1973. 6.; Brick: Common brunt clay building bricks; 1077:1992 (Fifth Revision). 7.; Light Gauge structural steel section; 811:1987. 8.; Collapsible gate:; 10521:1983. 9.; Marble: Block, slab, and tile; 1130:1969. 10.; Ceramic tile/ Glazed tile:; 777:1970; Spartek/Kajaria/ Orient/Somany. 11.; Vitrified porcelain tile; Granamite/ diomoite/Kajaria (kerogress). 12; Viny flooring; Tuskar/poly floor Armstrong. 13; Aluminum section: For door window; 1984:1961; Indal/Beco/OEL. 14; Wooden Flush door:; 2202(part2):1983; Greenply/Century/ Mayer. 15; Cement based paint:; Snowcemplus/ Durocem. 16; Acrylic Emulsion; ICI/ Berger/Asian Paints. 17; Deistemper; ICI/ Berger/Asian Paints. 18; Synthetic Enamel; ICI/ Berger/Asian Paints. 19; Water proofing Admixture; 2645 and 9103; SIKA /Forsrock/ Laticrete. 20; Glass/Glazing glass; Modi guard/saintgobain. Specification of Materials. 1; Sanitary Fitting : Hindware/ParryWare. 2; CI pipe & fitting: IS 3989 of Jayaswals /NECO Ltd /Similar approved.. 3; C.P fittings : Jaquar/ESSESS/GEM brand. 4; G.I. Pipes: Tata make (medium quality). 5; G.i. Fittings:'R'/HB/KS brand with ISI marked. 6; G.M Valves :Leader/zoloto brand. 7; SW pipes: Sonall/Hind brand. 8; CI Cover& frame: Local brand good quality. 9; Paint : Jhonson & Nicolson/ICI brand/Asian Paint. 10; Kitchen Sink: Nirall or similar approved.. 11; Mirror: Golden flsh or similar approved.. 12; Sluice Valve: Upadhya or similar approved.. Specification for Materials, Measurement & Workmanship as per CPWD & ISI.